1 × 3 Packet Router

Interview Question & Answer Preparation Guide

Verilog HDL  •  RTL Design & Verification

Prepared for Chaitanya Roy

Interviewer: senior engineer, ams OSRAM (Austria)

# How to use this guide

A senior interviewer with decades of experience rarely wants a recital — they probe whether you understand why each choice was made and what breaks if you change it. For every answer below, lead with the one-sentence version, then be ready to go deeper if asked. The italic “How to say it” notes give you a natural spoken phrasing. Questions are grouped: project overview, architecture, the FSM, the datapath blocks, design trade-offs, verification, problems you solved, and RTL fundamentals a senior will almost certainly test.

# 1.  Project overview

Q.  In one or two sentences, what did you build?

A.  A synchronous 1-to-3 packet router in Verilog. It accepts a byte-stream packet on a single input, reads the destination address from the packet header, buffers the payload in one of three FIFOs, and verifies the packet with a parity check before the destination reads it out.

How to say it in the room:  “It’s a single-master, three-channel packet switch — one input port, three output channels, with buffering and error detection built in.”

Q.  What is the packet format?

A.  Three parts streamed byte by byte. The header is one byte: the lower two bits are the destination address and the upper six bits are the payload length. Then the payload bytes, and finally a one-byte parity that closes the packet.

- data_in[1:0] → channel select (00, 01, 10; 11 is invalid)

- data_in[7:2] → payload length

- pkt_valid goes high for the whole packet and drops to mark the last data byte before parity.

Q.  What are the top-level inputs and outputs?

A.  Inputs: clock, resetn (active-low), data_in[7:0], pkt_valid, and read_enb_0/1/2. Outputs: data_out_0/1/2[7:0], valid_out_0/1/2, error, and busy. A packet goes in on data_in; each channel pulls its packet out with its own read_enb.

# 2.  Architecture & dataflow

Q.  Walk me through the block-level architecture.

A.  Five modules wired together by router_top, which has no logic of its own — it is pure structural instantiation:

- router_fsm — the controller. An 8-state machine that generates every control signal.

- router_reg — the datapath register: assembles the byte sent to the FIFO, generates parity, and flags errors.

- router_sync (synchronizer) — routes the write-enable to the correct FIFO, produces valid_out, and runs the soft-reset timeout.

- router_fifo ×3 — one 16×9 FIFO per output channel, buffering data until each channel reads it.

How to say it in the room:  “The FSM is the brain in the middle; the register and synchronizer are its two hands; the three FIFOs are the mailboxes.”

Q.  Trace what happens from the first byte to read-out.

- pkt_valid rises; the FSM in DECODE_ADDRESS reads addr[1:0] and checks the target FIFO is empty.

- The register latches the header byte; the synchronizer asserts write_enb for only that FIFO.

- Payload bytes stream through dout into the FIFO; parity is XOR-accumulated as they pass.

- The last byte is the packet’s parity; the FSM compares it against the internal parity and sets error if they differ.

- valid_out goes high; the destination pulls bytes out with read_enb until the FIFO empties.

Q.  Why did you split control and datapath into separate modules?

A.  Separation of concerns. One central FSM owns all sequencing and timing, so the control is easy to reason about and verify; the datapath blocks (register, FIFO) stay simple, combinational-plus-register, and reusable. It also keeps the FSM free of wide datapaths, which keeps its timing clean.

# 3.  The FSM (deep dive)

Q.  Name the states and what each does.

- DECODE_ADDRESS — idle; read address, wait for a valid packet.

- LOAD_FIRST_DATA — latch the header byte.

- LOAD_DATA — stream payload into the FIFO.

- LOAD_PARITY — capture the incoming parity byte.

- CHECK_PARITY_ERROR — compare parity, set the error flag, reset internal registers.

- FIFO_FULL_STATE — FIFO filled mid-packet; stall and hold the byte.

- LOAD_AFTER_FULL — resume loading once space frees up.

- WAIT_TILL_EMPTY — target FIFO still holds a previous packet; wait for it to drain.

Q.  How many always blocks, and what is in each?

A.  Three. A sequential always @(posedge clock) for the present-state register (with reset and the soft-reset timeout override); a combinational always @(*) for next-state logic; and the output logic as continuous assign statements decoded from the present state. That makes it a Moore machine — outputs depend only on the current state.

Q.  Moore or Mealy? Why does it matter here?

A.  Moore — the outputs are pure functions of the present state, so they’re glitch-free and easy to time. A Mealy machine would let outputs react to inputs one cycle earlier but risks combinational glitches on the control lines feeding the FIFOs. For a control path driving memory writes, the clean, registered Moore outputs are the safer choice.

Q.  How do you guarantee there are no inferred latches in the next-state logic?

A.  Two habits. First, the combinational block assigns a default (next_state = DECODE_ADDRESS;) at the top before the case. Second, always @(*) builds the sensitivity list automatically so no input is missed. Every path assigns next_state, so the synthesizer never needs a latch to hold it.

How to say it in the room:  “Default assignment plus a fully-specified case — that’s my anti-latch discipline.”

Q.  What is the WAIT_TILL_EMPTY state for?

A.  It prevents two packets from being interleaved in one FIFO. If a packet is addressed to a channel whose FIFO still holds an unread previous packet, the FSM parks in WAIT_TILL_EMPTY until that FIFO empties, then starts loading. Without it, a fast sender could corrupt the boundary between packets.

Q.  Explain the FIFO_FULL / LOAD_AFTER_FULL pair.

A.  If the FIFO fills while a packet is still streaming, the byte that couldn’t be written would be lost. So the FSM moves to FIFO_FULL_STATE and the register stashes that byte in an internal full_state_byte register. When the reader frees a slot, LOAD_AFTER_FULL pushes the held byte in and resumes. No data is dropped.

Q.  What is the busy signal and when is it asserted?

A.  busy tells the source the router can’t accept new header bytes right now. It’s high in every state except DECODE_ADDRESS and the very first load cycle — i.e., whenever the FSM is mid-packet, stalled on a full FIFO, waiting, or checking parity. It’s the router’s back-pressure/handshake to the sender.

# 4.  Datapath blocks — FIFO, Register, Synchronizer

Q.  Why is the FIFO 9 bits wide when the data is 8 bits?

A.  The 9th (top) bit stores lfd_state — it tags the first byte of a packet. Inside the FIFO you need to know where each packet begins so the read side can recover the length and packet boundary. Each location holds { lfd_state, data[7:0] }.

Q.  How do you generate the full and empty flags?

A.  With 5-bit pointers for a 16-deep FIFO: four address bits plus one extra wrap (MSB) bit. empty when read and write pointers are exactly equal; full when the addresses match but the wrap bits differ — meaning the writer has lapped the reader once. The extra bit is what lets you distinguish full from empty when both pointers point at the same location.

How to say it in the room:  “The wrap bit is the classic trick — same address, different MSB means full; identical means empty.”

Q.  How is parity generated and checked?

A.  As the packet streams, the register XOR-accumulates every header and payload byte into internal_parity. At the end, the incoming parity byte is captured into pkt_parity. Once parity_done is set, if pkt_parity != internal_parity the error flag is raised. It’s a cheap one-byte checksum — minimal hardware, catches single-bit and many multi-bit errors.

Q.  What does the synchronizer actually do — is it a CDC synchronizer?

A.  No — despite the name it is not a two-flop clock-domain-crossing synchronizer; the whole design is single-clock. Here it’s the routing/housekeeping block: it decodes the latched address to drive exactly one write_enb[2:0], produces valid_out_x = ~empty_x, muxes the active FIFO’s full flag back to the FSM, and runs the soft-reset timeout counters.

How to say it in the room:  “I’d flag the naming up front — it’s a routing block, not a metastability synchronizer, since everything runs on one clock.”

Q.  Explain the soft-reset timeout.

A.  Each channel has a counter. If a FIFO holds data but its channel doesn’t read for 30 clock cycles, the synchronizer asserts soft_reset for that channel, which clears its FIFO and returns the FSM to idle. It’s a livelock guard: a stuck or absent reader can’t block the whole router forever.

# 5.  Design decisions & trade-offs

Q.  Why parity instead of CRC?

A.  Parity is the lightest possible check — one byte, one XOR chain, no extra latency. A CRC catches more error patterns but costs more logic and a wider trailer. For this design the spec calls for a byte-level checksum; parity meets it with minimal area. If the channel were noisier or the spec demanded stronger detection, I’d move to a CRC.

Q.  Why a depth of 16 and a 30-cycle timeout — are those tunable?

A.  The FIFO depth is a parameter (DEPTH = 16), so it’s easy to change; 16 balances buffering against area for the expected packet sizes. The 30-cycle timeout is a chosen constant that’s comfortably longer than a normal read cadence but short enough to recover quickly. Both are design points I could justify or re-tune against real traffic.

Q.  What happens on address 2'b11?

A.  It’s invalid — there’s no fourth channel. The decode logic treats it as no valid destination: write-enable stays 000 and the FSM doesn’t start a load. So a bad address simply isn’t accepted rather than corrupting a channel.

Q.  Is this design synthesizable? Any simulation-only constructs?

A.  The RTL is synthesizable — clocked always blocks, resets, case/if, continuous assigns, a parameterized memory array. The 8'hzz high-impedance drive on the FIFO output models a shared/tri-stated read bus and is the one construct to call out; on an FPGA you’d typically replace tri-state with a mux. The testbenches use tasks, $dumpvars, and delays, which are simulation-only by design.

# 6.  Verification

Q.  How did you verify the design?

A.  Bottom-up. Every module has its own testbench in tb/ — FIFO, register, synchronizer, and FSM verified in isolation first, then a top-level testbench drives a full packet through the integrated router. Each testbench uses reusable tasks (initialise, reset, stimulus) to stay readable, and dumps a .vcd waveform I inspect in GTKWave.

Q.  How did you test the FSM specifically?

A.  Scenario-driven. The FSM testbench defines named scenario tasks (s1, s2, …), each walking a specific path through the state graph — the normal DECODE→LFD→LD→LP→CPE flow, the FIFO-full branch through FIFO_FULL_STATE and LOAD_AFTER_FULL, and the WAIT_TILL_EMPTY branch. That way every state and transition is exercised deliberately, not by luck.

Q.  How do you know parity checking works?

A.  I tested both directions: a clean packet where the sent parity matches, expecting error = 0, and a packet with a deliberately corrupted parity byte, expecting error = 1. Proving the negative case matters as much as the positive one.

Q.  This is directed testing. How would you take it further?

A.  I’d add constrained-random stimulus with randomized addresses, lengths, and reader back-pressure; a scoreboard/reference model to check output packets automatically instead of by eye; functional coverage on states, transitions, and the full/empty/timeout corners; and assertions (SVA) for invariants like ‘never write when full’ and ‘one-hot write_enb.’ That’s the natural move from a directed Verilog bench to a UVM-style environment.

How to say it in the room:  “I’m comfortable saying the current bench is directed; I’d layer randomization, a scoreboard, coverage, and assertions on top for sign-off.”

# 7.  Problems I faced & how I solved them

Problem:  Data lost when the FIFO filled mid-packet

Solution:  I added the FIFO_FULL_STATE / LOAD_AFTER_FULL states plus a full_state_byte holding register. The byte that can’t be written is parked and re-injected the instant a slot frees up — nothing is dropped.

Problem:  Two packets could land in the same FIFO

Solution:  I introduced WAIT_TILL_EMPTY so the FSM refuses to start a new packet for a channel until that channel’s FIFO is fully drained.

Problem:  A slow or absent reader could block the router forever

Solution:  The 30-cycle soft-reset counter in the synchronizer self-heals a stuck channel, so one dead reader can’t deadlock the whole router.

Problem:  Packet boundaries were lost inside the FIFO

Solution:  I widened the FIFO word to 9 bits and tagged the first byte with lfd_state, so the read side can recover where each packet begins and its length.

Problem:  Risk of inferred latches in next-state logic

Solution:  Default next_state assignment plus a fully-specified case under always @(*) — every branch defined, clean synthesis, no latches.

# 8.  RTL fundamentals a senior will test

Q.  Blocking vs non-blocking assignments — which did you use and why?

A.  Non-blocking (<=) in every clocked/sequential block, blocking (=) in combinational always @(*) blocks. That’s the standard rule: non-blocking models the parallel register updates of real flip-flops and avoids simulation races; blocking gives the immediate, ordered evaluation you want for combinational logic.

Q.  Why is reset active-low (resetn), and is it synchronous or asynchronous here?

A.  Active-low is an industry convention — most standard-cell flops have an active-low reset and it’s robust to glitches on power-up. In this design the reset is tested inside always @(posedge clock), so it behaves synchronously — sampled on the clock edge. Asynchronous assert / synchronous de-assert (reset synchronizer) would be the next refinement for a real chip.

Q.  What’s the difference between resetn and soft_reset in your design?

A.  resetn is the global hardware reset that clears the entire router. soft_reset is a targeted, self-generated recovery: it clears just one channel’s FIFO after the timeout, without disturbing the other two channels or the rest of the design.

Q.  Is there any clock-domain crossing in this design?

A.  No — it’s fully synchronous on a single clock, so there’s no metastability risk and no CDC synchronizer needed. If the three read channels ran on independent clocks, then each FIFO would become a true asynchronous FIFO and I’d Gray-code the pointers and double-flop them across the boundary.

How to say it in the room:  “Single clock today; if the readers were on other clocks I’d Gray-code the FIFO pointers and add two-flop synchronizers.”

Q.  Why Gray code for asynchronous FIFO pointers?

A.  Because only one bit changes per increment, so when you sample the pointer in the other clock domain you can only ever be off by that single bit — you never latch a wildly wrong value mid-transition. With binary pointers, multiple bits change at once and a mis-sample could read garbage.

Q.  What sets your maximum clock frequency?

A.  The longest combinational path between registers — here that’s the next-state logic and the parity XOR/compare feeding the flops. To push fmax I’d pipeline or balance those paths, but for this project I focused on functional correctness rather than timing closure.

Q.  If the payload length field says one thing but the packet is a different length, what happens?

A.  The FSM is driven by pkt_valid and the parity position, not by trusting the length field blindly for control — pkt_valid dropping marks the last data byte, and the next byte is parity. The length in the header is available on read-out via the tagged first byte. A mismatch would show up as a parity error, which is exactly the kind of corruption the check is there to catch.

# 9.  Closing questions to ask them

Asking one sharp question signals engagement. A few that fit ams OSRAM (sensors, LEDs, mixed-signal):

- What clocking and reset strategy does the team standardize on across blocks?

- How much of your verification is directed vs. UVM/constrained-random, and where is the team heading?

- For mixed-signal parts, how do the digital and analog timing/verification flows meet?

- What would a new graduate own in the first six months on this team?